lINSTRUMENTAL METHODS OF ANALYSIS- BP701T

UNIT : 1 UV Visible spectroscopy

CLASS: 5  Deviations of Beer Lamberts law, solvent effects

TOPIC: DEVIATIONS OF BEER LAMBERT’S LAW

The plot of absorbance versus concentration is expected to be a straight line passing by origin. Thus, this is not always true; there are certain limitations. A law does not carry for all species under every condition. Several a times instead of a straight line, a curvature within the plot might be observed as shown in Figure.  An upward curvature, curve (a), is called as positive deviation and the downward curvature, curve(c), since negative deviation.

Fig. 5.1: Beer-Lambert law plots; the curvatures show deviations from the law

A few of the factors responsible for the deviation from Beer's law are as follows.

| Complexation, Association or Dissociation | Concentration of the Analyte |
| --- | --- |
| Hydrogen Ion Concentration | Non-monochromatic Nature of the Radiation |
| Presence of Electrolytes |  |

Complexation, Association or Dissociation:

Some salts have a tendency to form complexes whose colours are different from those of the simple compounds. For example, the colour of cobalt chloride changes from pink to blue due to the following complex formation.

2 CoCl2   ↔   Co[CoCl4]

pink                            blue

The degree of complex creation increases along with increase in concentration, therefore, Beer's law does not hold at high concentrations. Same discrepancies are found while the absorbing solute dissociates or associates within solution since the nature of the species in solution depends on the concentration.

Concentration of the Analyte:

As per Beer and Lambert's law, the plot of absorbance versus the concentration of absorbing substance should be a directly line while ε and b are constant. The path length could always be held constant but there are a few factors that affect ε and it is found which at high concentration, ε is not constant. Thus, at higher concentrations (>10-3 mol dm3) there might be deviation from the law.

Temperature

The temperature is not considered as a significant factor since ordinarily the measurements are made at a constant temperature. Therefore, changes in temperature sometimes might shift ionic equilibrium and the absorptivity. For instance, the colour of acidic ferric chloride solution changes from yellow to reddish brown on heating because of change in λmax and absorptivity.

Hydrogen Ion Concentration:

There are a number of substances whose ionic state in solution is greatly influenced by the presence of hydrogen ions. For instance, the aqueous solution of potassium dichromate includes the chromate ion-dichromate ion equilibrium as shown below:

2CrO72- + 2H + ↔ Cr2O72- + H2O

Chromate ion            Dichromate ion

( λ max,375 nm)           ( λ max 350,450 nm)

The chromate ion has a single λmax at 375 nm whereas dichromate ion has two peaks in the spectra; λmax at 350 and 450 nm. The location of equilibrium depends on the pH of the solution and yellow color of solution changes to orange on increasing the concentration of hydrogen ions. Thus, the results of the determination of chromate ion ( CrO72-) concentration will depend on the pH. Therefore, it is imperative in which substances, whose color is influenced through change within hydrogen ion concentration, must be studied under the condition of similar pH.

In a few cases, two absorbing species are in equilibrium and have a general value of absorptivity at a certain wavelength. For instance, in case of bromothymol blue the absorption spectra at different pH values are different.  Thus, at wavelength of 501 nm, we see which all species have similar molar absorptivity. Thus, no matter to what extent does one species change within the other, there is no change in the total absorption. Like a wavelength is known as isosbestic point. At this wavelength the Beer's law holds, by the measurements have low sensitivity. Therefore, such wavelengths should be prevented for the quantitative work.

Non-monochromatic Nature of the Radiation:

In sequence for the Beer's law to carry, it is essential that monochromatic light is used. Therefore, most spectrophotometers and all filter photometers, discussed later employ a finite group of frequencies. The huger the bandwidth of radiation passed through the filter or other dispersing devices, a greater will be the apparent deviation of a system from adherence to Beer's law.

Presence of Electrolytes:

The presence of small amounts of colorless electrolytes which do not react chemically with the colored components does not affect the light absorption as a rule. However, large amounts of electrolytes may affect the absorption spectrum qualitatively as well as quantitatively. This is because of the physical interaction among the ions of the electrolyte and the colored ions or molecules. This interaction results in a deformation of the later, thus causing a modification in its light absorption property.

SOLVENT EFFECTS

Solvent should not absorb any radiation in the area being investigation. It should be less polar so that it interacts with the solute molecule as little as possible. Most frequently: Ethanol 95 percent It is inexpensive, has a high dissolving power, and does not absorb radiation beyond 210nm.

The solvent has a significant impact on the spectrum's quality and form. The absorption spectrum of a pharmaceutical drug is largely determined by the solvent used to solubilize the molecule. In one solvent, a medication may absorb a maximum amount of radiation energy at a specific wavelength, but only partially at the same wavelength in another solvent. For example, acetone in n-hexane has a maximum absorption wavelength of 279nm. Acetone in water has a maximum wavelength of 264.5nm