INSTRUMENTAL METHODS OF ANALYSIS- BP701T

UNIT : 2 Atomic absorption  spectroscopy

CLASS:8

TOPIC: APPLICATIONS, ADVANTAGES & DISADVANTAGES

APPLICATIONS

The Atomic absorption spectrometry technique is one of the most important instruments for detecting trace (ppm and ppb) metallic elements.

This approach is used for the quantitative determination of approximately 60 metals. Atomic absorption spectrometry is useful in all areas of chemical analysis. The atomic absorption spectroscopy method is well-established in analytical chemistry, ceramics, mineralogy, biochemistry, metallurgy, water supply, medicine, and soil analysis.

### I. Quantitative analysis

Quantitative analysis depends on determining the amount of radiation absorbed by the sample. This measurement provides the number of absorbing atoms in the light path. However, the concentration of the element in the sample cannot be calculated simply from the number of absorbing atoms in the atomizer. In practice, quantitative measurements are usually dependent on calibration.

Calibration curves are generated using a known concentration solution of the sample element. The sample is atomized similarly to the standard, and its concentration is calculated using the calibration curve. Atomic absorption spectroscopy is used for the quantitative determination of various metal elements.

### II. Metallurgical and inorganic analysis ;

The elemental composition of minerals and rocks provides important information on the commercial viability of mining activities in areas that have been studied. After mining, the ores and minerals must be examined for composition for refining activities to be efficient. Similarly, trace metal analysis is extremely useful in the search for oil and water deposits.

Atomic absorption spectrometry has been used to analyze trace metals in geological, biological, metallurgical, glass, cement, engine oil, marine sediments, pharmaceutical, and atmospheric samples. It is also used to determine trace elements in the ground, surface water, rocks, ores, minerals, and so on. Atomic absorption spectroscopy analyzes light alloys almost completely. Atomic absorption spectroscopy is also used to determine Co, Cr, Mg, Mn, Pb, Zn, and other elements in iron and steel.

### III. Pollution monitoring

Monitoring for trace metal pollution in the atmosphere and industrial effluents, oceans, rivers, and lakes is critical for determining the safety of living beings.

Atomic absorption spectrometry is utilized in pollution analysis, such as lead in the atmosphere and arsenic in ground water.

### IV. Food and Beverages

In the food and beverage industry, it is used for the analysis of various element concentrations in wine, beer, and fruit drinks. Atomic absorption spectroscopy is also used for the determination of various types of contamination in food.

### V. Medical pathology

- Ca, Mg, Na, and K levels in blood serum and plasma are determined by using Atomic absorption spectrometry.

•Atomic absorption spectroscopy is used in blood analysis for the determination of Na, K, Fe, Cu, Zn, and Pb.

• Atomic absorption spectroscopy is used in hair and nail analysis for the determination of Co, Zn, Mg, Pb, Cu, and Fe.

- Atomic absorption spectroscopy is utilized in the analysis of metals in food.

- Atomic absorption spectroscopy is utilized for metal analysis in petroleum products and lubricating oils.

### VI. Pharmaceutical  industry

Trace metal analysis is critical in formulation development, catalyst efficiency, and dose limit determination. Most elements have a helpful role up to specific prescribed limitations, but their impacts are hazardous beyond those limits. In the pharmaceutical industry, atomic absorption spectroscopy is used for the determination of trace elements present in drugs.

### VII. Biochemical analysis

Atomic absorption spectroscopy is used to determine Na and K levels in biological fluids, as well as Cd and Zn levels in industrial employees’ urine. In forensic labs, trace metal analysis gives useful information on specimens such as stomach contents for food poisoning, paint chips, fibers, and hair strands gathered at a crime scene.

### VIII. For material development

Composition and trace metals have a significant impact on common material characteristics such as hardness, brittleness, grain size, crystallinity, and amorphous nature. Trace metal analysis can provide useful information on the performance attributes of such materials.

### IX. Agriculture

Atomic absorption spectroscopy is used to determine the presence of metallic elements in soil such as Ca, Sr, and Co. Additionally, it is utilized in animal feed analysis for Zn, Cu, Mn, Ca, K, Na, and other elements. It is also used for analysis of animal fertilizer to determine the concentration of  Ca, Cu, Mg, Fe, and other elements.

### X. Selected determination by atomic absorption spectroscopy

- Mg and Ca concentrations in tap water (acetylene air flame)

- Vanadium present in lubricating oil

- Trace elements (Zn, Cu, Ni, Cd) in polluted soil

- Tin in canned fruit juice.

- Chromium in steel.

- Arsenic in soil and water by hydride formation.

- Arsenic in soil and water by hydride generation.

Advantages of AAS over FES

1) Spectral hindrance is very rare in AAS which is common in FES

2) AAS is not affected by temperature

3) FES is more sensitive for Al, Ba, Ca, Ho, In, K and Ln, whereas heavy metals are effectively detected by AAS.

4) AAS is very rapid technique and concentration less than 1 ppm can be determined by AAS. Atomic Absorption Spectroscopy

Disadvantages of AAS

1) Different hallow cathode lamps are required for different elements. Thus scope of AAS is limited.

2) The elements which form oxide on heating are not effectively estimated by AAS.( Al, W, Mo, Ti, Si, V etc.)

3) Sample required is in liquid form.

4) Common anions (Chloride, sulphate, carbonate) affect the efficiency of analysis.

5) Metal solution in organic solvent is preferred over aqueous solution.