INSTRUMENTAL METHODS OF ANALYSIS- BP701T

UNIT : 2 Nephlometry & turbidometry

CLASS:10

TOPIC:  EFFECT OF CONCENTRATION, APPLICATIONS

CHOICE OF METHOD

The choice of method i.e., Nephlometry or Turbidimetry depends on the concentration of suspension. When the concentration of suspension is less, scattering of light is less and hence can be accurately measured at 90oor any convenient angle. This makes nephlometry as the choice in low concentrated suspensions.

When the concentration of suspension is more, since scattering is also more, only transmitted light (unscatterd light) can be measured at 180o. This makes turbidimetry as the choice of method in high concentrated suspensions.

EFFECT OF CONCENTRATION AND WAVELENTH ON  SCATTERING OF LIGHT

Concentration

The intensity of transmitted light is expressed using an equation similar to that of Beer –Lamberts law i.e.,

P= poe-Tb      Where,

p= power of transmitted light

Po= power of incident beam

T=turbidity or turbidity co-efficient

b= path length

Tb =logpo/p

‘T’ was found to be proportional to the concentration of suspended particles

Hence, as T=Kc, kcb = logpo/p

Wavelength

It is expressed by the following equation,

T=S/λt

Where,

T=Turbidity

S = Constant for a given system

Λ =Wavelength

T=depends on size of particle and is ‘4’ when particle size is smaller than wavelength.

APPLICATIONS OF TURBIDIMETRY AND NEPHELOMETERY

Turbidimetry and nephlometry can be used in gaseous liquid or even transparent solid samples.

1. Turbidimetry and nephlometry are used in the study of air and water pollution. In air, dust and smoke are measured whereas in water, turbidity is measured.

2. The main uses of nephlometry and turbidimetry are the determination of sulphate as BaSO4, chloride as AgCl, fluoride as CaF2, cyanide as AgCn, calcium as oxalate, carbonate as BaCO4 and Zinc as ferrocyanide.

3. They are of higher importance in water treatment plants, sewage work, power and

steam generating plants, in beverage bottling industry, Paper & pulp industry,

Petroleum refineries and in pharmaceutical industry.

4.  In Water analysis, they are used to determine clarify and for understanding the

efficiency of treatment process.

5.  Turbidimetry is used to analyze turbidity in sugar products and citrus juices.

6.  Nephelometry helps to determine the content of carbonate as BaCo4, Cl  as AgCl, F

as CaF2, and Cyanide as AgCN.