INSTRUMENTAL METHODS OF ANALYSIS- BP701T

UNIT : 3  Introduction to Chromatography

CLASS:7

TOPIC:  PAPER ELECTROPHORESIS

CONTENTS:

- Introduction

- Types of electrphoresis

- Paper electrophoresis

- Components

- Types of paper electrophoresis

- Working & Detection

- Factors affecting electrophoresis

- Advantages

- Disadvantages

- Applications

Types of electrophoresis

- Zone Electrophoresis

a)Paper Electrophoresis

b)Gel Electrophoresis

c)Thin Layer Electrophoresis

d)Cellulose acetate Electrophoresis

- Moving Boundary Electrophoresis

a) Capillary Electrophoresis

b) Isotachophoresis

c) Isoelectric Focusing

d) Immuno Electrophoresis

Zone Electrophoresis 
It involves the migration of the charged particle on the supporting media. 

Paper, Cellulose acetate membrane, Starch, Gel, Poly acrylamide. 

Components separated are distributed into discrete zone on the support media.  Supporting media is saturated with buffer solution, small volume of the sample is applied as narrow band.

- Paper electrophoresis

- Separation of mixture is based on charge and size of the molecule.

- Migration of charged particles takes place through paper under the influence of electric field.

- The paper strip used must contain at least α-cellulose at a percent of 95% and should have a slight adsorption capacity.

- Paper does not contain charges that interfere with the separation of compounds.

- Practical aspects of PE:

- Whatman filter paper, cellulose acetate filter paper or chromatography paper.

- Prior to use, the paper is washed with distilled water followed by 0.1M HCl or 0.01M EDTA to remove impurities.

- A power pack provides a stabilised DC and has controls for both voltage output (0-500V) and current output (0- 150 mA).

- An electrophoretic cell- contains the electrodes, buffer reservoir, a support for paper and a transparent insulating cover.

Types of paper electrophoresis

Based on design of instrument, it is of two types:

1. Horizontal paper electrophoresis

2. Vertical paper electrophoresis