INSTRUMENTAL METHODS OF ANALYSIS- BP701T

UNIT : 4 GC and HPLC

CLASS: 9

TOPIC: GAS CHROMATOGRAPHY

DERIVATIZATION

- Derivatization reactions are meant to transform an analyte for detectability in Gas Chromatography (GC) or other instrumental analytical methods.

- Derivatization in GC analysis can be defined as a procedural technique that primarily modifies an analytes functionality in order to enable chromatographic separations.

- A modified analyte in this case will be the product, which is known as the derivative. The derivative may have similar or closely related structure, but not the same as the original non-modified chemical compound.

- Volatility of sample is a requirement for GC analysis. Derivatization will render highly polar materials to be sufficiently volatile so that they can be eluted at reasonable temperatures without thermal decomposition or molecular re-arrangement.

- Understanding the chemistry of the analytes, derivatizing reagents used in sample preparation, and the detailed functionality of Gas Chromatography are important to get reliable results.

- For GC analysis, compounds containing functional groups with active hydrogen such as -SH, -OH, -NH and -COOH are of primary concern because of the tendency of these functional groups to form intermolecular hydrogen bonds.

Objectives for derivatization

- The following outlined objectives among others can be achieved by application of proper derivatization procedures;

- Improvement of resolution and reduce tailing of polar compounds which may contain – OH, –COOH, =NH, –NH2, –SH, and other functional groups.

- Analysis of relatively nonvolatile compounds.

- if compounds are thermolabile

- Ito reduce tailing and improve seperation factor.

- Stabilization of compounds for GC analysis.

These Are Two Types:

PRECOLUMN DERIVATISATION : To improve the properties of the sample for seperation by column.

(ex: phenols converted to less polar compounds by using reagents like BSA Bis trimethyl silyl acetamide reagent)

POSTCOLUMN DERIVATISATION: to improve response shown by detector (their ionisation or affinity towards electrons increases)

Types of derivatization reactions

Derivatization reactions used for gas chromatography (GC) fall into three general reaction types namely;

Silylation

Silylation is the most prevalent derivatization method as it readily volatizes the sample and therefore very suitable for non-volatile samples for GC analysis.

Silylation is the introduction of a silyl group into a molecule, usually in substitution for active hydrogen such as dimethylsilyl [SiH(CH3)2], t-butyldimethylsilyl [Si (CH3)2C(CH3)3] and chloromethyldimethylsilyl [SiCH2Cl(CH3)2].

Alkylation

Alkylation is mostly used as the first step for further derivatizations or as a method of protection of certain active hydrogens in a sample molecule. It represents the replacement of active hydrogen by an aliphatic or aliphatic-aromatic (e.g., benzyl) group in process referred to as esterification.

Acylation

Derivatization by acylation is a type of reaction in which an acyl group is introduced to an organic compound.  In the case of a carboxylic acid, the reaction involves the introduction of the acyl group and the loss of the hydroxyl group. Compounds that contain active hydrogens (e.g., -OH, -SH and -NH) can be converted into esters, thioesters and amides, respectively, through acylation.

Acylation is also a popular reaction for the production of volatile derivatives of highly polar and in volatile organic materials.

Acylation also improves the stability of those compounds that are thermally labile by inserting protecting groups into the molecule.

Advantages

- GC is the method of choice for the separations of volatile materials due to

- Very speed (typically in minutes)

- High resolution capability

- Sensitive , easy detecting ppm and often ppb

- Non destructive, making possible on-line coupling

- Highly accurate in quantitative analysis, typical RSDs of 1-5 percent.

- Disadvantages

- Limited to volatile samples

- Fairly difficult for large, preparative samples and

- Requires elaborate instruments, such as mass spectroscopy for confirmation of peak identity

- APPLICATIONS

- Qualitative Analysis – by comparing the retention time or volume of the sample to the standard / by collecting the individual components as they emerge from the chromatograph and subsequently identifying these compounds by other method.

- Quantitative Analysis- area under a single component elution peak is proportional to the quantity of the detected component/response factor of the detectors.

- Volatile Oils, official monograph gives chromatography profile for some drugs. E.g. to aid distinction between anise oil from star anise and that from Pimpinelle anisum

- Separation of fatty acids derived from fixed oils

- Miscellaneous-analysis of foods like carbohydrates, proteins, lipids, vitamins, steroids, drug and pesticides residues, trace elements

- Pollutants like formaldehyde, carbon monoxide, benzene, DDT etc.,.

- Dairy product analysis- rancidity

- Separation and identification of volatile materials, plastics, natural and synthetic polymers, paints, and microbiological samples

- Inorganic compound analysis.