PHYSICAL PHARMACEUTICS-I – BP302T

UNIT: 4(COMPLEXATION AND PROTEIN BINDING)

CLASS: 31

TOPIC:  METHODS OF ANALYSIS

# Methods of Analysis of Complexation

The study and analysis of complex formation between drugs and complexing agents is essential to determine the stoichiometry, stability constant, and nature of complexes. Various physical, chemical, and spectroscopic methods are used for the analysis of complexation.

## 1. Phase Solubility Analysis

### Principle

This method is based on the change in solubility of a drug in the presence of increasing concentrations of a complexing agent.

### Procedure

- Excess drug is added to solutions containing different concentrations of complexing agent

- Solutions are shaken until equilibrium is reached

- Drug concentration in solution is determined

### Importance

- Determines complex stoichiometry

- Calculates stability constant

- Widely used for cyclodextrin complexes

## 2. Spectroscopic Methods

### (a) UV–Visible Spectroscopy

Principle:
Complex formation causes a change in absorbance or wavelength (λmax).

Applications:

- Determination of stability constant

- Identification of charge-transfer complexes

### (b) Infrared (IR) Spectroscopy

Principle:
Complexation alters vibrational frequencies of functional groups.

Applications:

- Identification of binding sites

- Confirmation of complex formation

### (c) Nuclear Magnetic Resonance (NMR)

Principle:
Chemical shift changes indicate complex formation.

Applications:

- Structural elucidation

- Inclusion complex confirmation

## 3. Potentiometric Method

### Principle

Based on changes in pH or electrode potential due to complex formation.

### Applications

- Determination of formation constants

- Used for metal–ligand complexes

## 4. Conductometric Method

### Principle

Complex formation results in a change in electrical conductivity of the solution.

### Applications

- Detection of ionic complexes

- Determination of stoichiometry

## 5. Polarographic Method

### Principle

Complexation changes the current–voltage curve of an electroactive species.

### Applications

- Analysis of metal complexes

- Determination of stability constants

## 6. Dialysis Method

### Principle

Free drug diffuses through a semipermeable membrane, while complexed drug does not.

### Applications

- Measurement of free and bound drug

- Protein binding and macromolecular complexes

## 7. Solubility Method

### Principle

Complex formation alters the apparent solubility of the drug.

### Applications

- Simple method to detect complexation

- Used in formulation development

## 8. Chromatographic Methods

### Types

- Thin Layer Chromatography (TLC)

- High Performance Liquid Chromatography (HPLC)

### Principle

Complex formation alters retention time or mobility.

### Applications

- Separation and identification of complexes

## 9. Thermal Analysis

### Techniques

- Differential Scanning Calorimetry (DSC)

- Thermogravimetric Analysis (TGA)

### Principle

Complex formation causes changes in melting point and thermal behavior.

### Applications

- Confirmation of inclusion complexes

- Stability studies

## 10. Job’s Method of Continuous Variation

### Principle

Total molar concentration of drug and complexing agent is kept constant while their ratio is varied.

### Applications

- Determination of stoichiometry of the complex