PHYSICAL PHARMACEUTICS-I – BP302T

UNIT: 5(pH, BUFFERS AND ISOTONIC SOLUTIONS)

CLASS: 39

TOPIC:  PH DETERMINATION ELECTROMETRIC AND CALORIMETRIC

## Introduction

Determination of pH is essential in pharmaceutical sciences because pH affects the stability, solubility, bioavailability, and therapeutic effectiveness of drugs. Accurate pH measurement is especially important for injectable, ophthalmic, and liquid oral formulations. pH can be determined by electrometric and colorimetric methods.

## Electrometric Method of pH Determination

### Principle

The electrometric method is based on the measurement of electrical potential (EMF) developed between two electrodes when immersed in a solution. The potential difference is directly related to the hydrogen ion concentration of the solution.

### Electrodes Used

- Glass electrode – sensitive to hydrogen ions

- Reference electrode – calomel electrode or silver–silver chloride electrode

### Construction of Glass Electrode

- Thin glass bulb filled with buffer solution of known pH

- Internal silver–silver chloride electrode

- Glass membrane selectively permeable to H⁺ ions

### Working

- When the glass electrode is immersed in the test solution, an electrical potential develops due to difference in H⁺ ion concentration across the glass membrane

- This potential is measured using a pH meter

- The meter converts EMF into pH value

### Procedure

- Calibrate the pH meter using standard buffer solutions

- Rinse electrodes with distilled water

- Immerse electrodes in the test solution

- Record pH reading after stabilization

### Advantages

- Highly accurate and precise

- Suitable for colored and turbid solutions

- Widely used in pharmaceutical analysis

### Limitations

- Expensive equipment

- Requires calibration and maintenance

- Glass electrode is fragile

## Colorimetric Method of pH Determination

### Principle

The colorimetric method is based on the color change of pH indicators, which are weak acids or bases that exhibit different colors at different pH values.

### Indicators Commonly Used

| Indicator | pH Range | Color Change |
| --- | --- | --- |
| Methyl orange | 3.1–4.4 | Red to yellow |
| Methyl red | 4.4–6.2 | Red to yellow |
| Bromothymol blue | 6.0–7.6 | Yellow to blue |
| Phenolphthalein | 8.3–10.0 | Colorless to pink |

### Procedure

- Add a few drops of suitable indicator to the test solution

- Observe the color produced

- Compare with standard color chart or buffer solutions

- Determine approximate pH

### Advantages

- Simple and economical

- No sophisticated equipment required

- Useful for quick estimation

### Limitations

- Less accurate

- Not suitable for colored or turbid solutions

- Subjective interpretation of color

## Comparison Between Electrometric and Colorimetric Methods

| Electrometric Method | Colorimetric Method |
| --- | --- |
| Highly accurate | Less accurate |
| Uses pH meter | Uses indicators |
| Suitable for all solutions | Not suitable for colored solutions |
| Expensive | Economical |
| Quantitative | Semi-quantitative |

## Pharmaceutical Importance

- Ensures proper drug stability

- Prevents irritation in ophthalmic and injectable preparations

- Helps in buffer selection

- Maintains product quality and safety