PHARMACEUTICAL ENGINEERING (BP304T)

UNIT: 4 (FILTRATION, CENTRIFUGATION)

CLASS: 33

TOPIC: INDUSTRIAL EQUIPMENT

Centrifuges of different types are available to meet the specific requirements of the slurry and the end product. In addition, experimental conditions are established by considering the factors affecting the process. Some of them are:

- Nature of slurry		 ▸Densities of the materials to be separated

- Flow of feed or slurry 	 ▸ Bottle (centrifuge) size

- Centrifuge dimensions	 ▸ Centrifugation time

Perforated Basket Centrifuge

In this centrifuge, a basket is mounted above a driving shaft. Such an arrangement is described as under-driven. Conversely, if the basket is suspended from a shaft, it is described as over-driven. These are used for batch processes. An under-driven bowl centrifuge is described below.

Principle: Perforated basket (bowl) centrifuge is a filtration centrifuge. The separation is through a perforated wall based on difference in the densities of solid and liquid phases. The bowl contains a perforated side-wall. During centrifugation, the liquid phase passes through the perforated wall, while solid phase is retained in the bowl. The solid is removed after cutting the sediment by a blade after stopping the centrifuge.

Construction: The construction of a under-driven perforated basket centrifuge is shown in Figure. It consists of a basket, made of steel (sometimes covered with vulcanite or lead) or copper or monel or any other suitable metal. The basket material of construction should be such that it offers the greatest resistance to corrosion. The basket may have a diameter of 0.90 metres and a capacity of 0.085 metre cube. The diameter of the perforations should be selected based on the size of crystals to be separated. In case, the size of perforations is than that of the particles, a filter cloth is employed.

Construction of perforated basket centrifuge.

The basket is suspended on vertical shaft and is driven by motor using suitable power systems such as belt pulleys, water turbines and electric motors. The basket may require about 5-kilowatt power for starting and 2-kilowatt power for running. Sometimes, steel hoops are used externally to strengthen the basket. Surrounding the basket, a casing (stationary) is provided which collects the filtrate and discharges it at the outlet.

Working: The material (feed) is kept in the basket when the basket is stationary. The amount of material should be optimum, otherwise. during centrifugation it puts a great strain on the basket. The loading of material must be done to give even distribution. Power is applied to rotate basket and maximum speed must be attained quickly. The basket runs at 1000 revolutions per minute. The power required for running is small compared to the power required for starting and bringing it to the full speed.

During centrifugation, the liquid passes through the perforated wall, while the solid phase retains in the basket. The liquid leaves the basket and is collected at the outlet. The cake is then spun to dry as much as possible. Sometimes higher speeds are used so that the cake will be completely dried. After a definite period of time, the power is turned off. By applying a brake, the centrifuge is stopped. The basket is brought to rest. The solid cake is cut using a blade and then unloaded manually.

Uses:

- Perforated basket centrifuge is extensively used for separating crystalline drugs (such as aspirin) from the mother liquor. Free flowing product can be obtained because mother liquor is re- moved completely.

- It is also used for removing unwanted solids from a liquid. For example, precipitated proteins are removed from insulin.

- Sugar crystals are separated using perforated basket centrifuge.

Advantages:

- The centrifuge is very compact and it occupies very little floor space.

- It can handle slurries with a high proportion of solids and even those having paste like consistency.

- The final product has very low moisture content.

- In this method, the dissolved solids are separated from the cake.

- The process is rapid.

Disadvantages:

- The entire cycle is complicated resulting in considerable labour costs.

- It is a batch process.

- If the machine is adapted for prolonged operation, there is considerable wear and tear of the equipment. On prolonged operation, the solids may form hard cake, due to the centrifugal force, which is difficult to remove simultaneously.

Variants: For the separation of fine crystals, the perforations of 3 millimetre in diameter are sufficient. But for finely ground materials, the basket is lined with fine meshed gauze or with a cloth supported on a coarser gauge.

MCQ Each carry 1 mark

- Which is the example of batch type centrifuge?

  - Conical disc centrifuge

  - Horizontal centrifuge

  - Perforated basket centrifuge

  - Supercentrifuge

- Which one of the following is an under-driven centrifuge?

  - Horizontal continuous centrifuge

  - Perforated basket centrifuge

  - Semi-continuous centrifuge

  - Conical disc centrifuge

- Centrifuges are used for the analysis of dosage forms in terms of one of the following stabilities.

  - Chemical stability

  - Photostability

  - Physical stability

  - Thermal stability

- 17. Which one of these is the gravitational constant in SI units (m/s 2 )?

  - 0.09807

  - 0.9807

  - 9.807

  - 98.0

- Which one of the following is a filtration centrifuge?

  - Conical disc centrifuge

  - Continuous horizontal centrifuge

  - Perforated basket centrifuge

  - Super-centrifuge

Each carry 2 marks

- Write the applications of perforated basket centrifuges.

- What is meant by under-driven filtration type basket centrifuges?

Each carry 5 marks

- Classify industrial centrifuges. Write construction and working of a perforated basket centrifuge.

Each carry 10 marks

- Discuss construction and working of a discontinuous centrifuge for solid separation