PHARMACEUTICAL ENGINEERING (BP304T)

UNIT: 4 (FILTRATION, CENTRIFUGATION)

CLASS: 37

TOPIC: CLASSIFICATION OF FILTRATION EQUIPMENTS FILTER PRESS

Based on application of external force:

 Pressure filter – Plate and frame filter press, metafilter

 Vacuum filters – Filter leaf

 Centrifugal filters

Based on operation of filtration

 Continuous filtration - Discharge and filtrate are separated steadily and uninterrupted

 Discontinuous filtration - Discharge of filtered solid is intermittent. Filtrate is

removed continuously. Operation should be stopped to collect solid. Based on nature of filtration
 Cake filters - Remove large amount of solids
 Clarifying filters - Remove small amounts of solids
 Cross-flow filters - Feed of suspension flows under pressure at fairly high velocity
across the filter medium
EQUIPMENTS [1, 2]
Plate and frame filter press
Principle:
 Mechanism is surface filtration.
 The slurry enters the frame by pressure and flows through filter medium.
 The filtrate is collected on the plates and send to outlet.
 A number of frames and plates are used so that surface area increases and
consequently large volumes of slurry can be processed simultaneously with or without

Filter press

Construction

 The Filter press is made of two types of units, plate and frames.

 Usually made of aluminium alloy.

 Sometimes, these are also lacquered for protection against corrosive chemicals and

made suitable for steam sterilization.

Frame-

 It contains a open space inside wherein the slurry reservoir is maintained for filtration

and an inlet to receive the slurry.

 It is indicated by two dots in description.

 Frames of different thickness are avail

t is selected based on the thickness of cake formed during filtration.

 Optimum thickness of frame should be chosen.

Plate

 The plate has a studded or grooved surface to support the filter cloth and an outlet.

 It is indicated by one dot in description.

 Plate supports the filter medium, receiving the filtrate and outlet.

 The filter medium usually cloth is interposed between plate and frame.

 Plate, filter medium, frame, filter medium and plate are arranged in sequence and

clamed to a supporting structure.

 It is normally described by dots as 1.2.1.2.1 so on.

 A number of plates and frames are employed so that the filtration area is as large as

necessary.

 Number of filtration units are operated in parallel.

 Channels for slurry inlet and filtrate outlet can be arranged by fitting eyes to the plates

and frames, these join together to form a channel.

 In some types only one inlet channel is formed, while each plate is having individual

outlets controlled by valves.

Working

Working can be divided into two steps

1. Filtration operation

 Frame- marked by 2 dots

 Plate – marked by 1 dot

 Slurry enters the frame from the feed channel and passes through the filter medium on

the surface of the plate

 The solid forms a filter cake and remain in the frame

 The thickness of the cake is half of the frame thickness, because on each side of frame

filtration occurs

 Thus two filter cakes are formed , which meet eventually in the centre of the frame

 The filtrate drains between the projections of the surface of the plate and escape from

the outlet

 As filtration proceeds, the resistance of the cake increases and filtration rate decrease

 At a certain point process is stopped and press is emptied and cycle is restarted.

2. Washing of cake (if desirable)

 When washing of cake is also required modified plate and frame filter is used.

 For this purpose an additional channel is included called as washing plate and are

identified by 3 dots

n the half of the washing plate, there is a connection from wash water cannel to the

surface of plate.

 The sequence of arrangement of plates and frames can be represented by dots as

1.2.3.2.1.2.3.2.1 so on (between 1 and 1, 2.3.2 must be arranged.

Procedure for washing the press

• Filtration proceeds in the ordinary way until the frames are filled with cake.

• To wash the filter cake, the outlets of washing plates are closed.

• Wash water is pumped in the washing channel. The water enters through the inlets on

the surface of washing plate.

• Water passes through the filter cloth and enters frame which contains the cake. Then

water washes the cake, passes through the filter cloth and enters the plate down the

surface

 

• Finally washed water escapes through the outlet of that plate

Special provisions:

• Any possible contamination can be observed by passing the filtrate through a glass

tube or sight glass from the outlet on each plate.

• This permits the inspection of quality of filtrate. The filtrate goes through the control

valves to an outlet channel.

• The filtration process from each plate can be seen. In the event of broken cloth, the

faulty plate can be isolated and filtration can be continued with one plate less.

Uses:

• Sterile filtrate can by obtain by using asbestos and cellulose filter sheet (for this,

whole filter press and filter medium have been sterilized previously).

• Filtration of viscous liquid can also be done by incorporating heating/cooling coils in

the press.

Advantages

• Construction of filter press is very simple and a variety of materials can be used.

• Provide large filtration area in relatively small floor space.

• The capacity being variable according to thickness of frames and number used.

• Sturdy construction permits the use of considerable pressure difference. (2000

Kilopascals normally used)

• Efficient washing of cake is possible.

• Operation and maintenance is easy.

• It produces dry cake in form of slab.

Disadvantages

• It is a batch filter, so it is a time consuming.

• The filter press is an expensive filter, the emptying time, the labour involved, and the

wear and tear on the cloths resulting in high costs.

• Operation is critical, as the frames should be full, otherwise washing is inefficient

and the cake is difficult to remove.

• The filter press is used for slurries containing less about 5 % solids In view of the

high labour costs , it is most suitable for expensive materials e.g. the removal of

precipitated proteins from insulin liquors