PHARMACEUTICAL ENGINEERING (BP304T)

UNIT: 4 (FILTRATION, CENTRIFUGATION)

CLASS: 38

TOPIC: FILTER LEAF META FILTER SWEETLAND FILTER

Filter leaf

Principle:

• It is an apparatus consisting of a longitudinal drainage screen covered with a filtercloth.

• The mechanism is surface filtration and acts as sieve or strainer.

• Vacuum or pressure can be applied to increase the rate of filtration.

Construction:

• The leaf filter is consisting of a frame enclosing a drainage screen or grooved plate.

• The frame may be any shape circular, square or rectangular.

• The whole unite being covered with filter cloth.

• The outlet for the filtrate connects to the inside of the frame through suction

Working

• The filter leaf is immersed in the slurry

• Vacuum system is connected to the outlet

• The slurry passes through the filter cloth

• Finally filtrate enters the drainage canal and goes through the outlet into receiver

• Air is passed to flow in reverse direction which facilitates removal of cake

Use:

• Use for the filtration of slurry which do not contain high solid content, about 5%, i.e.

dilute suspensions.

Advantages

• Simplest form of filter used for batch process.

• A number of units can be connected in parallel to increase the surface area of

filtration.

• Pressure difference can be obtained either with vacuum or using pressure up to the

order of 800 kilopascals.

• Labour costs for operating the filter leaf are fairly moderate.

• The efficiency of washing is high.

• The slurry can be filtered from any vessel.

• The cake can be washed simply by immersing the filter in a vessel of Water.

Sweetland filter (variant of filter leaf)

• An alternative method is to enclose the filter leaf in a special vessel into which the

slurry is pumped under pressure.

• A number of leaves are connected to a common outlet, to provide a large area for

Filtration

Metafilter

Principle:

• Mechanism is surface filtration.

• In this, metal rings contain semicircular projections, which are arranged as a nest to

form channels on the edges.

• This channel offers resistance (strainer) to the flow of solids (coarse particles).

• The clear liquid is collected into receiver from the top.

Construction

• Metafilter consists of a series of metal rings.

• These are threaded so that a channel is formed on the edges.

• It contains a grooved drainage column on which a series of metal rings are packed.

• These rings are usually made up of stainless steel and have dimensions of about 15.0

mm internal diameter and 22.0 mm external diameter.

• Each metal ring has a number of semicircular projections (0.8 mm in thickness) on

one side of surface.

• The projections are arranged as a nest to form channels on the edges.

• These rings are tightened on the drainage column with a nut.

• Metafilters are also known as edge filters.

Working

• Filters are placed in a vessel

• Slurry is pumped under pressure or occasionally by applying reduced pressure to the

outlet side

• Slurry passes through the channels formed on the edges between the rings

• The clear liquid rises up and collected from the outlet into receiver

• For separation of fine particles, a bed of suitable materials such kieselguhr is first

built up.

• The pack of rings serves essentially as a base on which the true filter medium is

supported.

Uses

Metafilters can be used for-

• Clarification of syrups

• Filtration of injection solutions

• Clarification of insulin liquors

• Filtration of viscous liquids can be achieved by applying pressure.

Advantages

• Can be used under high pressures, without any danger of bursting the filter medium.

• Running cost is low, as separate filter medium is not used.