PHARMACEUTICAL ENGINEERING (BP304T)

UNIT: 4 (FILTRATION, CENTRIFUGATION)

CLASS: 39

TOPIC: CATRIDGE FILTER, ROTARY DRUM FILTER, MEMBRANE FILTER, SIETZ FILTER

Cartridge filter

Principle:

• It is a thin porous membrane in which pre filter and membrane filter are combined in

a single unit.

• The filtration action is mainly sieve like and particles are retained on the surface.

Construction:

• It has cylindrical configuration made with disposable or changeable filter media.

• Made up of either plastic or metal.

• Consist of two membrane filters (sieve like) made of polypropylene: prefilter and

actual filter for filtration.

• A protective layer surrounds them.

• The cartridge is housed in a holder and a number of cartridges can be placed in a same

housing.

• The housing is closed with the lid.

• Housing has provisions for slurry inlet and outlets.

Working:

• Slurry is pumped into cartridge holder

• It passes through cartridge filter unit by straining

• The clear liquid passes through the centre

• Moves up to collect through outlet

Uses:

• Particularly useful for preparation of particulate free solutions for parenterals and

ophthalmic uses.

• This filter holder will process 1000 – 15000 litres of sterile solution per hour.

Advantages:

• Autoclaving can be done for sterile operations due to stainless steel construction.

• Cartridges with self cleaning devices are advantageous.

• Rapid disassembling as well as reusing of filter medium is possible.

• Cartridges are not brittle, when they are dry.

• Used as in-line continuous filtration, this reduces handling of solution. It minimizes

chances of contaminations.

Disadvantages:

• A number of manufactures provide the components, which are generally not

interchangeable between suppliers.

• Cost of disposable elements offsets the labour saving in terms of assembly and

cleaning of cartridge clarifiers.

Rotary drum filter2

Principle:

• Slurry filtered through sieve like mechanism on the rotation drum surface, under the

condition of vacuum.

• In addition compression, drying (using hot air), and removing the filter cake (using

knife) are possible.

Construction:

• It consists of a metal cylinder mounted horizontally.

• The drum may be up to 3 meters in diameter and 3.5 meters in length and gives

surface area of 20 meter square.

• The curved surface being a perforated plate, supporting a filter cloth.

• Internally, it is divided into several sectors and a separate connection is made between

each sector and a special rotary valve.

Fig 7 – Rotary drum Filter

Working

• The drum is dipped into the slurry and vacuum applied to the outlet, which is

connected to the filtrate receiver.

• When the cake has formed, the cake drained or partially dried by vacuum.

• The drum is sprayed with water to wash the cake.

• Retaining the vacuum connection drains the cake and produces partial dryness then,

removed by a doctor knife.

• When the solids of the slurry are too much that the filter cloth becomes blocked with

the particles, a pre-coat filter may be used.

• A pre-coat of filter aid is deposited on the drum prior to the filtration process.

Uses

• The rotary filter for continuous operation on large quantities of slurry.

• Suitable for slurry contains considerable amounts of solids in the range 15-30%.

• Examples of pharmaceutical application includes the collection of calcium carbonate,

magnesium carbonate, and starch.

• The separation of the mycelium from the fermentation liquor in the manufacture of

antibiotics.

Advantages

• The rotary filter is automatic and is continuous in operation, so that the labour costs

are very low.

• The filter has a large capacity, so it is suitable for the filtration of highly concentrated

solutions.

• Variation of the speed of rotation enables the cake thickness to be controlled.

• Pre-coat of filter aid could used to accelerate the filtration rate.

• Filter has large surface area.

Disadvantages

• The rotary filter is a complex piece of equipment, with many moving parts and is very

expensive.

• In addition to the filter itself, some accessories are connected, e.g., a vacuum pump,

vacuum receivers, slurry pumps and agitators are required.

• The cake tends to crack due to the air drawn through by the vacuum system, so that

washing and drying are not efficient.

• Being a vacuum filter, the pressure difference is limited to 1 bar and hot filtrates may

boil.

• It is suitable only for straight- forward slurries

Membrane filters [2]

Membrane filters act as a barrier to separate contaminants from water, or they remove the

particles contaminating the water. Reverse osmosis, ultrafiltration, and nanofiltration all use a

membrane in their different filtration processes.

Construction

• Membrane filters are made of thin and flat membranes of cellulose derivatives, such

as, cellulose acetate and cellulose nitrate.

• These filters are brittle when in dry condition and can be stored for an indefinite

period.

• The filters are between 50 and 150 μ thick and are available in sizes upto 60 cm2 .

Fig 8- Membrane filter

Working

• A membrane filter has 400 to 500 million pores per square centimetre of filter surface.

• The pores are absolutely uniform in size and occupy about 80% of filter volume.

• To avoid rapid clogging of a membrane, pre-filtration is often required.

• The selection of a membrane filter for a particular application depends on the particles

to be removed.

Uses

• These filters are mainly used for sterilization of both aqueous and oily liquids.

• The membrane filters cannot be used for filtration of organic solvents, such as

alcohols, ketones, esters and chloroform.

SEITZ FILTER [3]

Principle

• It is based on filtration of asbestos pad filter disc

Construction

• It consists of two parts. Lower part fitted with a perforated plate over which

compressed asbestos pad is placed.

• Upper part has a value through which pressure can be applied.

• Both parts joined together by winged nuts.

• A valve is present on the upper part through which vacuum is applied

• The asbestos pads may yield alkali and cause precipitation of alkaloids

• It may shed fibres into the filtrate and absorb drug from solution.

Fig 9- Seitz Filter

Advantages:

• No risk of contaminating the filtrate.

• Apparatus is very simple to use.

• For viscous solution they are more suitable.

Disadvantages:

• Asbestos may shed loose fibers.

• Pad may absorb sufficient amount of medicament