PHYSICAL PHARMACEUTICS II BP(403T) UNIT II (RHEOLOGY)

CLASS 13

TOPIC: ROTATIONAL VISCOMETERS

# Brook field viscometer

- Is a rotational viscometer of searle type that is popular in the quality control laboratories of pharmaceutical manufacture

- A number of spindles of various geometries, including cylinders t-bars and a cone-plate configuration are avialable

- Will provide scientific rheological data For newtonian and non newtonian liquids

- Useful for pastes and semisolid materials

- Various models are avialable to measure low, medium and high viscosity applications

- Standard operating procedure:

- The viscometer is turned on and allow standing, must be auto zero.

- After the few seconds the screen appears which indicates 2 digits.

- Now the key should be pressed.

- The screen displays the instruction to remove spindle.

- After removing the spindle and pressing the key the instrument begins. It is auto zeroed.

- After approximately 15 second the screen displays the instruction to replace spindle

- Attach the spindle to viscometer by screwing them on the lower shaft using left hand thread.

- Press select spindle key and up and down arrow keys. When the desired code is displayed release the arrow key.

- To select spindle, first press either up and down key which cause the area to show current speed, press the set speed key for adjusting the speed.

- Insert center of this spindle in the test material until the fluid level is at the immersion groove on the spindle shaft.

- Tilt the spindle slightly while immersing to avoid air entrapment.

- To measure high viscosity choose a small spindle and to slow speed if the chosen spindle/speed result in torque above 100% and then reduce thevalue.

- Allow time for the individual reading to stabilize, record the value.

- Press the motor on/off/escape key to turn off motor

- The time mode allows the user to record the reading for fixed period of time or until a set torque value is attained.

- Then enter/auto turnkey allows determining maximum calculated viscosity possible with current spindle/speed.

- Pressing the up and down arrow keys will allow the viscometer data to be examine, pressing and other key ( except enter/ print key) will bring back normal display.

- The mains was turned off after use.

# Cone and plate viscometer (Rotational viscometer)

- Principle: The sample is placed on at the center of the plate, which is raised into the position under the cone.

- The cone is driven by variable speed

- motor and sample is sheared in the narrow

gap between stationary plate and rotating cone.

Rate of shear in rpm is increased & decrease by selector dial and	viscous traction or torque (shearing stress) produced on the cone

Viscosity for Newtonian system can be estimated by,

η = C T/V	eq.1 Where,

C = Instrument constant,

T = Torque reading & V = Speed of the cone (rpm)	Plastic viscosity determined by,

U = Cf	T – Tf / v

Yield value f = Cf × Tf

Tf	= Torque at shearing stress axis (extrapolate from linear portion of curve). Cf = Instrument constant

# Advantages of cone and plate viscometer

- Rate of shear is constant through out the sample being sheared any chance of plug flow is avoided

- The rate of shear at any diameter is the ratio of linear velocity Ώr to the gap width d

- G= Ώr/d cm/sec/cm the ratio	r/d is constant and is proportional to Ψ the angle between cone and plate in radians G= Ώ/ Ψsec-1

- Thus G is independent of radius of the cone

- The cone angle generally ranges from 0.3o to 0.4o

- The time saved in cleaning and filling and temperature stabilization

- Cone and plate requires very little amount of the sample i.e 0.2 to 0.3 ml(cup and bob requires 20 to 50ml)

- By means of suitable attached it is also possible to increase and then decrease rates of shear in a predetermined and reproducible manner

- The shear stress is plotted as a function of rate of shear on X-Y recorder

- Valuable aid in determining the area of hysteresis or thixotropic cofficients because it allows comparative studies to be run in a consistent manner