# Deformation of solids

- It is possible to change the shape or size (or both) of an object through the application of external forces

- It is described as strain.

- As deformation occurs internal inter molecular forces arise that oppose the applied force.

- If the applied force is not too great these internal forces are sufficient to completely resists the applied force and allow the object to assume a new equilibrium state and to return to its original state when the load is removed.

- A larger applied force may lead to a permanent deformation of the object or even to its structure failure.

- Depending upon the type of material, size and geometry of the object and the forces applied various deformation may result.

# Types of deformation

- With continued increased pressure, the particles will deform via one or more of the following mechanisms:

- 1.Brittle fracture

- 2.Elastic deformation

- 3.Plastic deformation

# Brittle fracture

- With brittle fracture, the particle structure fails and breaks into two or more pieces

- . A commonly used excipient that exhibits this behavior is dicalcium phosphate.

# Elastic Deformation

- It is reversible. when the forces are removed, the object tends to its original shape.

- Elastomers and shape memory metals such as nitinol exhibit large elastic deformation ranges as does rubber.

- Elasticity is non linear.

- Metals and ceramics show linear elasticity.

- Linear elastic deformation is governed by Hooke’s Law

- σ = E ε

- Where σ is applied stress, E is material constant called young’s modulus or elastic modulus and ε

is the resulting strain.

- The relationship indicates that the slope

of stress vs. strain curve can be used to find the elastic or young modulus E.

- The elastic range ends when the material reaches its yield strength.

- At this point plastic deformation begins.

- Pregelatinized starch and some active ingredients behave in this manner.

# Plastic Deformation

- It is irreversible. Object in plastic deformation range will first have undergone elastic deformation which is reversible

- so the object will partly return to its original shape.

- Soft thermoplastic materials have rather large plastic deformation range as do ductile metals

- such as copper, silver and gold.

Hard thermosetting plastics, rubber and ceramics have minimal plastic deformation ranges

- Under the tensile stress plastic deformation is

- characterized as

- Strain hardening region – material becomes stronger through the movement of atomic dislocations

- Necking region – reduction in cross sectional area of specimen. It begins after the ultimate strength is reached. Material can no longer withstand the maximum stress and strain in the specimen rapidly increases

- Fracture- indicates the end of the plastic deformation

A material that exhibits this behavior is microcrystalline cellulose. Breaking

  - If stress continues, it surpasses its ultimate strength

  - The ultimate strength is the greatest stress the object can withstand without breaking

The breaking point

  - For a brittle material, the breaking point is just beyond its ultimate strength

  - For a ductile material, after passing the ultimate strength the material thins and stretches at a lower stress level before breaking

# Significance of deformation

Using these evaluation techniques can help developers with:

  - selecting formulation ingredients;

  - designing tablets, including concavity and profile shape;

  - choosing punch-head options to provide longer or shorter consolidation or dwell times;

  - determining whether precompression and tapered dies are needed to exhaust entrapped air from the material in the die; and

  - reducing tablet press wear and tear from excessive compression forces and friction.

  - Planning for scale-up

  - Identifying material deformation characteristics at the development level allows developers to formulate a drug product to withstand the compression dynamics of a high-speed production tablet press.

  - The driving force for choosing a particular excipient is its functionality, such as

  - diluting the formulation, enhancing the API’s therapeutic performance, enhancing solubility, facilitating powder flow, reducing sticking and picking potential, and more.

  - However, understanding the mechanical characteristics of a drug product’s excipients under known tableting conditions can pay long-term dividends when the product is ready to be scaled up for high-speed production.