# CLASS 16 TOPIC:STRESS,STRAIN,MODULUS

Elastic Modulus

- The elastic modulus is the constant of proportionality between stress and strain

- For sufficiently small stresses, the stress is directly proportional to the strain

- The constant of proportionality depends on the material being deformed and the nature of the deformation

- The elastic modulus can be thought of as the stiffness of the material

- A material with a large elastic modulus is very stiff and difficult to deform

- s t r e s s = E l a s t i c m o d u l u s × s t r a i n

# Young’s Modulus

- Tensile stress is the ratio of the external force to the cross-sectional area

- SI units of stress are Pascals, Pa 1 Pa = 1 N/m2

- The tensile strain is the ratio of the change in length to the original length

- Strain is dimensionless

- The elastic modulus is called Young’s modulus F/A=YΔL/Lo

# Shear Modulus

- It defines the change in shape of an object caused by pushing its top and bottom surfaces in opposite directions.

- Shear stress is the force per unit area exerted on the top and bottom of solid.

- Shear strain is the distance the top surface has moved relative to the bottom surface divided by the perpendicular distance between them.

- Shear Modulus is the ratio of shear stress to shear strain and has a unit of N/m2.

- S = Shear stress / Shear Strain

= F/A/Δx/h

- Where

- F is applied force:

- A is area,

- Δx is the distance

- the top surface has moved relative to bottom surface, and h is perpendicular distance between the top and bottom surfaces.

Bulk	Modulus:

Volume Elasticity

- Bulk modulus characterizes the response of an object to uniform squeezing

- Suppose the forces are perpendicular to, and act on, all the surfaces

- Example: when an object is immersed in a fluid

- The object undergoes a change in volume without a change in shape

- Volume stress, ΔP, is the ratio of the force to the surface area

- This is also the Pressure

- The volume strain is equal to the ratio of the change in volume to the original volume