# PHYSICAL PHARMACEUTICS II BP(403T) UNIT IV (MICROMERITICS)

CLASS 34

## Topic: Air Permeability Method

- The principal resistance to the flow of a fluid such as air through a plug of compacted powder is the surface area of the powder.

- The greater is the surface area per gram of powder, Sw, the greater is the resistance to flow.

- Hence, for a given pressure drop across the plug, permeability is inversely proportional to specific surface.

- From equations Sw = αs/ƿ αvdvs it is then possible to compute dvs.

- A plug of powder can be regarded as a series of capillaries whose diameter is related to the average particle size.

- The internal surface of the capillaries is a function of the surface area of the particles. According to Poiseuille equation

- Where V is the volume of air flowing through a capillary of internal diameter d and length l in t seconds under a pressure difference of ΔP. The viscosity of the fluid air is η poise.

- The Kozeny Carman equation, derived from the Poiseuille equation, is the basis of most air permeability methods. Stated in one form, it is

- Where A is the cross-sectional area of the plug, K is a constant (usually 5.0 ± 0.5) that takes account of the irregular capillaries, and ε is the porosity, instrument is the Fisher sub sieve sizer

- In practice, the flow rate through the plug, or bed, is also affected by

- The degree of compression of the particles

  - The more compact the plug, the lower is the porosity, which is the ratio of the total space between the particles to the total volume of the plug.

- The irregularity of the capillaries.

  - The irregularity of the capillaries means that they are longer than the length of the plug and are not circular

  - From the values of A ,k, ε,ΔP(value obtained from manometer) η is the (viscosity of the fluid air) and v(the volume of the air that flowed in time t ,the specific surface can be calculated

  - The Kozney carman equation gives the effect of porosity on sw and dvs and dvs decreases with decrease of porosity

## Fisher sub sieve sizer

## Applications and advantages of Fisher sub sieve sizer

  - Simple instrumentation and speed of determination made it possible to control batch to batch variations

  - For standardization of powder drugs

  - Ex: Bephenium hydroxy napthoate(antihelmenthic drug) is stndardised by means of air permeability method.The drug is administered as suspension should posess a specific surface of 7000cm2/g as the specific surface reduced the activity of the drug also falls

  - For the determination of specific surface of Griseofulvin